Project “”

Signature

Date:				Checked By:

Prepared By:			Verified By:

Load Consideration

Load for Parking P1, P2, P3, P4, P5

Dead Load

Slab thickness = 200 mm

Dead load of slab		 			= 0.2 x 25 = 5 kN/m2

Slab thickness = 175 mm

Dead load of slab		 			= 0.175 x 25 = 4.375 kN/m2

Floor finish 			 			= 1.5 kN/m2

Parking load 						= 3 kN/m2

Sunk  = 150

= 0.15 x 18 = 2.7 kN/m2

2) Home store

Slab thickness = 200 mm

Dead load of slab		 			= 0.2 x 25 = 5 kN/m2

Slab thickness = 175 mm

Dead load of slab		 			= 0.175 x 25 = 4.375 kN/m2

Slab thickness = 150 mm

Dead load of slab		 			= 0.15 x 25 = 3.75 kN/m2

Floor finish 			 			= 1.5 kN/m2

Live Load 						= 4 kN/m2

Service Load 						= 5 kN/m2

A.H.U 							= 5 kN/m2

Sunk  = 150 						= 0.15 x 18 = 2.7 kN/m2

3) Cinema Hall

Slab thickness = 200 mm

Dead load of slab		 			= 0.2 x 25 = 5 kN/m2

Slab thickness = 175 mm

Dead load of slab		 			= 0.175 x 25 = 4.375 kN/m2

Slab thickness = 150 mm

Dead load of slab		 			= 0.15 x 25 = 3.75 kN/m2

Floor finish 			 			= 1.5 kN/m2

Live Load 						= 4 kN/m2

Service Load 						= 5 kN/m2

A.H.U 							= 5 kN/m2

Sunk  = 150 						= 0.15 x 18 = 2.7 kN/m2

4) Typical Floors

Slab thickness = 200 mm

Dead load of slab		 			= 0.2 x 25 = 5 kN/m2

Slab thickness = 150 mm

Dead load of slab		 			= 0.15 x 25 = 3.75 kN/m2

Slab thickness = 175 mm

Dead load of slab		 			= 0.175 x 25 = 4.375 kN/m2

Slab thickness = 150 mm

Dead load of slab		 			= 0.15 x 25 = 3.75 kN/m2

Floor finish 			 			= 1.5 kN/m2

Live Load 						= 2.5 kN/m2

Service Load 						= 5 kN/m2

Sunk  = 150 						= 0.15 x 18 = 2.7 kN/m2

Wall Load

200 mm THK						= 0.2 x 20 x (Floor Height – Beam Depth)

150 mm THK						= 0.15 x 20 x (Floor Height – Beam Depth)

100 mm THK						= 0.1 x 20 x (Floor Height – Beam Depth)

Glass Load (Glazing)

50 mm THK						= 0.05 x 25 x Floor Height

Earth quake loads are calculated as per seismic co-efficient method as suggested in IS 1893-(Part 1) - 2002.

Design seismic Base Shear

Design seismic base shear can be determined by the following expression

VB = Ah x W

Ah = Design horizontal acceleration spectrum value as per 6.4.2 using fundamental natural period Ta

Ta =  0.075h 0.75

h = Height of the building from natural ground level = 108 m

Ta = 0.075 x 108 0.75

Ta = 2.51

1) For rocky or hard soil sites

Sa / g  =	1 / 2.51	    =   0.39			---------------  (1) ( Cl.6.4.5)

2) For medium soil sites

Sa / g  =	1.36 / 2.51	    =   0.54			---------------- (2) ( Cl.6.4.5)

Design Spectrum

The design horizontal seismic co-efficient for a structure shall be determined by the following expressions.   ( As per cl. 7.8.2 & 7.6 )

Ah	=	 ZI 	Sa

2R	 g

Z	–	Zone factor for Zone II

–	Low Seismic Intensity				=	0.10

I	–	Importance factor				=	1.0

R	–	Response reduction factor for ordinary

Shear Wall with Moment resisting frames	=	3.0

Sa	–	Average response acceleration co-efficient which depends on soil

G		types, natural period and damping values.

T	–	Fundamental Natural period	=	0.075h 0.75

Where ‘h’ is the height of the building (108 meters)

Assuming medium soil and damping 5%.

1) For rocky or hard soil sites

A h   =	0.1        1       0.39     =    0.0065

2	3

2) For medium soil sites

A h   =	0.1        1       0.54     =    0.009

2	3

Seismic Enhancement

Seismic weight of the building

Total Dead Load		= 1269301kN

Total Live Load ( No reduction)	=  444457 kN

D.L + 50%L.L			= 1269301 + 0.5 x 444457

= 1529030 kN

1) For rocky or hard soil sites

Design seismic base shear

VB	= 	Ah x W

=	0.0065 x 1529030

=	9938 kN

= 	993.8 T

2) For medium soil sites

Design seismic base shear

VB	= 	Ah x W

=	0.009 x 1529030

=	13761 kN

=	1376.4 T

Distribution of design shear force

Vb x WiHi2

WiHi2

Qi =   Vb x WiHi2

WiHi2

| Floor | Wi ( kN ) | Hi ( M ) | WiHi2 x 103 | Qi ( kN ) |
| --- | --- | --- | --- | --- |
| 21 | 1342.5 | 108.925 | 15928300.2 | 1288.417 |
| 20 | 1342.5 | 105.325 | 14892829.9 | 1204.659 |
| 19 | 1342.5 | 101.725 | 13892157.3 | 1123.716 |
| 18 | 1342.5 | 98.125 | 12926282.2 | 1045.588 |
| 17 | 1342.5 | 94.525 | 11995204.8 | 970.2746 |
| 16 | 1342.5 | 90.925 | 11098924.9 | 897.7758 |
| 15 | 1342.5 | 87.325 | 10237442.7 | 828.0918 |
| 14 | 1342.5 | 83.725 | 9410758.03 | 761.2225 |
| 13 | 1342.5 | 80.125 | 8618870.98 | 697.1679 |
| 12 | 1342.5 | 76.525 | 7861781.53 | 635.928 |
| 11 | 1342.5 | 72.925 | 7139489.68 | 577.5029 |
| 10 | 1342.5 | 69.325 | 6451995.43 | 521.8925 |
| 9 | 1342.5 | 65.725 | 5799298.78 | 469.0968 |
| 8 | 1342.5 | 62.125 | 5181399.73 | 419.1159 |
| 7 | 1342.5 | 58.525 | 4598298.28 | 371.9496 |
| 6 | 1342.5 | 54.925 | 4049994.43 | 327.5981 |
| 5 | 1342.5 | 51.325 | 3536488.18 | 286.0614 |
| 4 | 1342.5 | 47.725 | 3057779.53 | 247.3393 |
| 3 | 1342.5 | 44.125 | 2613868.48 | 211.432 |
| 2 | 1342.5 | 40.525 | 2204755.03 | 178.3394 |
| 1 | 1342.5 | 36.925 | 1830439.18 | 148.0616 |
| CA | 1342.5 | 31.925 | 1368283.55 | 110.6785 |
| Hs | 1342.5 | 22.15 | 658660.706 | 53.2781 |
| As1 | 1342.5 | 16 | 343680 | 27.79977 |
| As2 | 1342.5 | 11.5 | 177545.625 | 14.36141 |
| As3 | 1342.5 | 7 | 65782.5 | 5.32105 |
| As4 | 1342.5 | 3.5 | 16445.625 | 1.330263 |
| As5 | 1342.5 | 0 | 0 | 0 |
|  |  |  |  | 13424 |

Dynamic Analysis

Dynamic analysis shall be performed to obtain the design seismic force, and its distribution to different levels along the height of the building and to the various lateral load resisting elements, for the following buildings:

- Regular buildings —  Those greater than 40 m in height in Zones IV and V and those greater than 90 m in height in Zones II and III.